API functions					On Entry				On Exit

- $00	System reset					A = Reset type			none

- $01	Input character 	(waits)		none					A = Character input

- $02	Output character 	(waits)		A = Character to output		A = Character output

- $03	Input status					none					NZ if char avail

- $04	Input line					A = Size, DE = Address		A = Length, DE = Address

- $05	Input line default				none					A = Length, DE = Address

- $06	Output line					DE = Address			none

- $07	Output new line				none					none

- $08	Get version details				none					Version info in A B C D E H L

- $09	Claim jump table entry			A = Number, DE = Address	none

- $0A	Delay in milliseconds			DE = Milliseconds			none

- $0B	Output message				A = Number				none

- $0C	Read jump table entry			A = Number				DE = Address

- $0D	Select console in/out device 		A = Device number			none

- $0E	Select console input device		A = Device number			none

- $0F	Select console output device		A = Device number			none

- $10	Input char from console device		E = Device number			A = Character, NZ flagged if OK

- $11	Output char to console device		A = Character, E = Device		NZ flagged if OK, else A = Char

- $12	Poll idle events				none					none

- $13	Configure idle events			A = Mode (0=Off, 1=On)		none

- $14	Timer 1 control (A x 1ms)			A = Period, DE = Address		none

- $15	Timer 2 control (A x 10ms)		A = Period, DE = Address		none

- $16	Timer 3 control (A x 100ms)		A = Period, DE = Address		none

- $17	Output port initialise			A = Port address			A = Output (zero)

- $18	Write to output port			A = Output byte			A = Output byte

- $19	Read from output port			none					A = Output byte

- $1A	Test output port bit			A = Bit (0 to 7)			A = Output masked

- $1B	Set output port bit				A = Bit (0 to 7)			A = Output byte

- $1C	Clear output port bit			A = Bit (0 to 7)			A = Output byte

- $1D	Invert output port bit			A = Bit (0 to 7)			A = Output byte

- $1E	Input port initialise				A = Port address			A = Input byte

- $1F	Read from input port 			none					A = Input byte

- $20	Test input port bit				A = Bit (0 to 7)			A = Input masked

- $21	Set baud rate				A = Device, E = Rate code		NZ flagged if OK

- $22	Execute command line			DE = Pointer to line			Z flagged if OK

- $23	Get pointer to command line		none					DE = Pointer to line

- $24	Skip delimiter in command line		DE = Pointer to line			A = Char, DE = Pointer

- $25	Skip non-delimiter in command		DE = Pointer to line			A = Char, DE = Pointer

- $26	Get Hex parameter from command	DE = Pointer				A = Status, DE = Ptr, HL = Value

- $27	Get current console I/O devices		none					D = Output, E = Input

- $28	Get top of free memory			none					DE = Top of free memory

- $29	Set top of free memory			DE = Top of free memory		none

- API Monitor Command:							API Assembler Instructions:

- API  <function number>  [<A>]  [<DE>]				LD 	C, <function number>

- result displayed:  <A>  <DE>						RST 	$30

Self-test (at reset)

Test output is via LEDs on the system’s status display port (eg. RC2014 or LiNC80 Digital I/O module)

Pass			Single sweep of lights followed by all lights off

Failed RAM		Continuous sweeping of lights

Failed serial		LED bit 0 stays on if serial module not found

Monitor command line interpreter

Flag and condition names

Flags:		set	clear

Zero		Z	NZ

Carry		C	NC

Negative	S	NS

Half carry	H	NH

Parity	even	Pa	NP

Subtract	N	NN

Conditions:

Zero		Z	zero set

Not zero	NZ	zero clear

Carry		C	carry set

Not carry	NC	carry clear

Negative 	M	sign set

Positive	P	sign clear

Even		PE	parity set

Odd		PO	parity clear

? / HELP						Display help

A 		[<address>]				Assemble instructions

API		<function>  [<A>]  [<DE>]	Call API function

B 		[<address>]				Breakpoint set or clear

BAUD		<device>  <rate code>		Set baud rate

CONSOLE	<device identifier>			Select console device

D 		[<address>]				Disassemble instructions

DEVICES						List devices detected

DIR							List files in the ROM

E 		[<address>]				Edit memory

F 		[<flag or condition name>]	Flags display or modify

FILL		<start>  <end>  <byte>		Fill memory

G 		[<address>]				Go to program

I 		<port> 				Input from port

M 		[<address>]				Memory display

O 		<port>  <data>			Output to port

R 		[<register name>]			Registers display or edit

RESET							Reset monitor

S 		[<address>] 			Step one instruction

Monitor ROM may also include additional commands, such as “BASIC”.

Jump Table

Baud rate codes

Baud rate		Rate codes

230,400		$1	$23

115,200		$2	$11

57,600		$3	$57

38,400		$4	$38

19,200		$5	$19

14,400		$6	$14

9,600			$7	$96

4,800			$8	$48

2,400			$9	$24

1,200			$A	$12

- $B	$60

- 300			$C	$30

- Either code can be used

- $00	Non-maskable interrupt handler

- $01	Restart $08, console character output

- $02	Restart $10, console character input

- $03	Restart $18, console input status

- $04	Restart $20, handler (not currently used)

- $05	Restart $28, breakpoint handler

- $06	Restart $30, applications programming interface (API) handler

- $07	Restart $38, mode 1 interrupt handler

- $08	Console input routine

- $09	Console output routine

- $0A	Reserved for get console input status

- $0B	Reserved for get console output status

- $0C	Idle event handler

- $0D	Timer 1 event handler

- $0E	Timer 2 event handler

- $0F	Timer 3 event handler

- $10	Device 1 input character 		default = serial port channel A

- $11	Device 1 output character 	default = serial port channel A

$12	Device 2 input character 		default = serial port channel B

Console devices

Console device	Identifiers

#1 (eg. SIO A	)	$1	$A

#2 (eg. SIO B	)	$2	$B

#3 (serial port 3)	$3

#4 (unassigned)	$4

#5 (unassigned)	$5

#6 (unassigned)	$6

- Either identifier can be used

- $13	Device 2 output character 	default = serial port channel B

- $14	Device 3 input character

- $15	Device 3 output character

- $16	Device 4 input character

- $17	Device 4 output character

- $18	Device 5 input character

- $19	Device 5 output character

- $1A	Device 6 input character

- $1B	Device 6 output character

Memory map

ROM	(minimum)	8 kbytes, 0x0000 to approx 0x1E00 used

RAM	(minimum)	32 kbytes assumed from 0x8000 to 0xFFFF, 0xFC00 to 0xFFFF used